DEAR VINCENT

BELOW IS MY DATA LOGGER PROGRAM FOR 1998

WE HAD THE 2 OMAFRA DATA LOGGERS CALIBRATED AND FIXED ETC

FOR FREE...I MAY USE THEM WITH 1 PROVISO, I GIVE THEM THE:

1. temperature

2. relative humidity

3. leaf wetness

DATA

SO NOW I HAVE 2 HMP35C’S (WHICH I DID NOT HAVE LAST YEAR)

AND 2 LEAF WETNESS SENSORS

AND 2 TIPPING RAIN BUCKETS

SO EACH DATA LOGGER WILL HAVE 1 HMP35C, 1 LEAF WETNESS SENSOR AND MAYBE 1 RAIN BUCKET

HERE’S MY QUESTIONS ABOUT THE PROGRAM BELOW

1. I used your old program from last year to figure out the commands for the leaf wetness sensors, last year we had two on one data logger and this year we have one...so i modified it, but I’m not sure if i was correct

2. i used short cuts to figure out what to do for the hmp35c plus looked at a 1990 program from omafra and modified them and am not sure it is correct

3. i have done nothing with the rain buckets yet

soooooooo please look over the program (I’ve interjected some points) and see if it looks good?

Program:

Flag Usage:

Input Channel Usage:

Excitation Channel Usage:

Continuous Analog Output Usage:

Control Port Usage:

Pulse Input Channel Usage:

Output Array Definitions:

*     1       Table 1 Programs

01: 60      Sec. Execution Interval

01:  P10      Battery Voltage

01: 1       Loc :

02:  P91      If Flag

01: 28      8 is reset

02: 30      Then Do

03:  P19      Signature

01: 2       Loc :

04:  P86      Do

01: 18      Set high Flag 8

05:  P95      End

DO I NEED THIS 21X REFERENCE TEMPERATURE?

AND IF YES, SHOULD IT BE 01:3 OR 01:8

I HAVE TWO PRINTED PROGRAMS AND ONE SAYS 3 AND ONE SAYS 8

You can put any number under location. This only determines where in memory it resides. It does not refer to wiring.

You don’t need a reference temperature because you are using a HMP35, not a thermocouple.

06:  P17      Panel Temperature

01: 3       Loc :

HERE IS THE COMMANDS FOR THE HMP35C...

07:  P11      Temp 107 Probe

01: 1       Rep

02: 1       IN Chan

03: 4       Excite all reps w/EXchan 4

04: 4       Loc :

05: 1       Mult

06: 0       Offset

08:  P86      Do

01: 41      Set high Port 1

09:  P22      Excitation with Delay

01: 1       EX Chan

02: 0       Delay w/EX (units=.01sec)

03: 15      Delay after EX (units=.01sec)

04: 0       mV Excitation

10:  P1       Volt (SE)

01: 1       Rep

02: 5       5000 mV slow Range

03: 2       IN Chan

04: 5       Loc :

05: 1       Mult

06: 0       Offset

11:  P86      Do

01: 51      Set low Port 1

Page 2   Table 1

12:  P89      If X<=>F

01: 5       X Loc

02: 3       >=

03: 100     F

04: 30      Then Do

13:  P30      Z=F

01: 100     F

02: 5       Z Loc :

DO I NEED A P95 END HERE?

YES.

HERE’S THE MODIFIED VERSION OF YOUR PROGRAM FROM LAST YEAR FOR THE LEAF WETNESS SENSORS

14:  P5       AC Half Bridge

01: 1       Rep

02: 15      5000 mV fast Range

03: 3       IN Chan

04: 3       Excite all reps w/EXchan 3

05: 2500    mV Excitation

06: 5       Loc :

07: 1       Mult

08: 0       Offset

You can’t store 2 values under the same memory location. Change LOC to a number not used anywhere else. (This is true for all sensors, you MUST give ONE location per sensor reading.

15:  P89      If X<=>F

01: 5       X Loc

02: 3       >=

03: 0.35    F

04: 12      Set high Flag 2

16:  P91      If Flag

01: 12      2 is set  high

02: 30      Then Do

17:  P30      Z=F

01: 1       F

02: 7       Z Loc :

18:  P95      End

WHY THE END ABOVE?

The END refers to the IF FLAG 2 HIGH DO

19:  P91      If Flag

01: 22      2 is reset

02: 30      Then Do

20:  P30      Z=F

01: 0       F

02: 7       Z Loc :

21:  P95      End

AGAIN WHY THE END ABOVE, DOESN’T THE P86 COMMAND ON THE NEXT PAGE BELONG TO THE ABOVE STUFF?

I’M REALLY GRASPING AT STRAWS HERE...

Again, a IF condition requires an END

The P86 below resets the FLAG 2 at each minute so that when leaf wetness is over it remains low

22:  P86      Do

01: 22      Set low Flag 2

23:  P92      If time is

01: 0       minutes into a

02: 15      minute interval

03: 10      Set high Flag 0 (output)

24:  P80      Set Active Storage Area

01: 1       Final Storage Area

02: 111     Array ID or location

Page 3   Table 1

25:  P77      Real Time

01: 1220    Year,Day,Hour-Minute

IS THE LOCATION BELOW CORRECT?

IN YOUR PREVIOUS PROGRAM IT WAS 9

26:  P71      Average

01: 1       Rep

02: 4       Loc

OK for air temperature

27:  P71      Average

01: 1       Rep

02: 7       Loc

this should be 5 for relative humidity…

28:  P92      If time is

01: 0       minutes into a

02: 1440    minute interval

03: 10      Set high Flag 0 (output)

29:  P80      Set Active Storage Area

01: 1       Final Storage Area

02: 333     Array ID or location

30:  P77      Real Time

01: 1220    Year,Day,Hour-Minute

31:  P70      Sample

01: 1       Rep

02: 1       Loc

32:  P70      Sample

01: 1       Rep

02: 2       Loc

33:  P95      End

WHAT’S ALL THE CRAP BELOW HERE?

Just that. CRAP.

Seriously, this is a very simple program. Longer program require tables 2 and 3 and subroutine and the rest of the crap below!!!

34:  P        End Table 1

*     2       Table 2 Programs

01: 0.0000  Sec. Execution Interval

01:  P        End Table 2

*     3       Table 3 Subroutines

01:  P        End Table 3

*     4       Mode 4 Output Options

01: 00      Tape/Printer Option

02: 00      Printer Baud Option

*     A       Mode 10 Memory Allocation

01: 28      Input Locations

02: 64      Intermediate Locations

Page 4   Mode C

*     C       Mode 12 Security (OSX-0)

01: 00      Security Option

02: 0000    Security Code

Page 5    Input Location Assignments (with comments):

Key:

T=Table Number

E=Entry Number

L=Location Number

T:  E:  L:

1:  1:  1:  Loc :

1:  3:  2:  Loc :

1:  6:  3:  Loc :

1:  7:  4:  Loc :

1: 10:  5:  Loc :

1: 13:  5:  Z Loc :

1: 14:  5:  Loc :

1: 17:  7:  Z Loc :

1: 20:  7:  Z Loc :